Full Performance Comparison Table

Secure-storage SoC compared with ECG Huffman + CBC-AES paper baseline

| Metric | ECG CBC-AES Paper [1] | This Work | Correctness | Improvement (%) |
| --- | --- | --- | --- | --- |
| Compression ratio | 35.015% | 29.87% | 100% | +14.69% |
| Space saving | 64.985% | 70.13% | 100% | +7.92% |
| PRD / byte match | 0.411 | 0 mismatch byte-exact RX | 100% | 0% |
| Compression time | ~3.8641 s | 1.056 ms TX comp-only | 100% | +99.97% |
| Decompression time | ~0.5818 s | 0.483 ms RX Huffman | 100% | +99.92% |
| Encryption time | ~2.7106 s | 29.6 us TX AES | 100% | +99.999% |
| Decryption time | ~3.0449 s | 69.2 us RX AES | 100% | +99.998% |
| Compression + encryption | ~6.5747 s | 1.065 ms TX path | 100% | +99.98% |
| TX/RX cycles | N/R | 53233 TX 24222 RX | 100% | 0% |
| TX/RX throughput | N/R | 6.828 MB/s TX-in 15.072 MB/s RX-out | 100% | 0% |

[1] A lossless compression and encryption mechanism for remote monitoring of ECG data using Huffman coding and CBC-AES, FGCS 2019. Paper platform: MATLAB 2018a on Windows 7 64-bit, i5 2nd Gen, 8 GB RAM. N/R = not reported. RTL timing is measured from performance counters at 50 MHz on five MIT-BIH records.